ARTICLE 6    OFF STREET PARKING / LOADING / DRIVEWAYS

SECTION 601   	GENERAL REQUIREMENTS

601.1	In districts with business, industrial, recreational or other non-single family residential uses - no building or structure shall be erected, substantially altered, changed in use, or any land used or changed in use unless adequately maintained off-street parking spaces have been provided, either in garages or open parking areas.

601.2	In districts with business, industrial, recreational or other non-single family residential uses - no building or structure shall be erected, substantially altered, changed in use, or any land used or changed in use unless off-street loading spaces have been provided.

601.3	A parking and loading plan shall not be required for single-family residential use. All other land uses shall submit a parking and loading plan to the Zoning Administrator as part of the application for a Zoning Permit. The parking and loading plan shall show:

A.	Boundaries of the property

B.	Parking spaces

C.	Loading areas

D.	Circulation patterns

E.	Drainage provisions

F.	Construction plans for any boundary walls or fences

G.	A screening plan, if applicable

H.	Location of any adjacent houses or other use buildings.

SECTION 602	OFF-STREET PARKING DESIGN STANDARDS

All parking facilities, including entrances, exits, maneuvering areas, and parking spaces shall be in accordance with the following specifications:

602.1	Parking Space Dimensions: All parking spaces shall be in accordance with the following design requirements:

602.2	Access: All parking spaces, except those required for single family use not fronting upon an arterial or collector street, shall have access to a public street in such a manner that any vehicle leaving or entering the parking area from or onto a public street or private interior driveway shall be traveling in a forward motion.

602.3 	Paving: Parking spaces and any driveway and other accessway required for vehicle access and circulation for any non-agricultural use other than for a single family dwelling shall be hard surface paved with asphalt or concrete.

602.4 	Drainage: All parking spaces, together with driveways, aisles, and other vehicular circulation areas shall be graded and drained so to dispose of surface water which might accumulate within or upon such area, and shall be designed to prevent the excess drainage of surface water on to adjacent properties, walkways, bikeways and public streets without causing damage or deterioration thereof.

602.5 	Barriers: Wherever a parking lot abuts a property line, then appropriate fencing, wheel stops, curbs, or other suitable structural barriers shall be provided in order to prevent any part of a parked vehicle from protruding beyond the property line it abuts.

602.6 	Screening: Screening of a non-residential use parking area that abuts any residential use property or residential zoning district shall be required as specified in Section 503.

602.7 	Access to Required Trash Containment Areas: Trash and/or garbage collection areas for commercial and industrial uses that provide such services, shall be enclosed on at least three sides by a solid wall or fence of at least 4 feet in height if such area is not enclosed in a building or other structure. Provisions for adequate vehicular access to and from an area for collection of trash and/or garbage shall be required.

602.8 	Other Uses Within Required Parking Areas: No motor vehicle repair work or service of any kind, other than for an emergency, shall be permitted in or associated with any off-street parking area. Display or sales of any merchandise within any parking area shall be permitted only by the Zoning Administrator in accordance with Section 513.2 F of this Resolution.

602.9 	Landscaping: All parts of open off-street parking areas which are unusable, either for parking or for traffic, shall be landscaped to prevent erosion and continuously maintained.

602.10 	Visibility: Access of driveways to and/or from a parking area shall be located so that any vehicle entering or leaving the parking area shall be clearly visible for an acceptable safe distance by any pedestrian or motorist approaching the driveway from a public or private street, walkway or bikeway, as applicable.

602.11 	Marking: Parking spaces shall be marked and maintained with painted lines, curb stones, or other manner determined suitable by the Board of Zoning Appeals.

602.12	Maintenance: The owner of a property used for a parking area shall maintain parking spaces and access ways thereto in good condition, without holes and free of dust, trash, and other debris.

602.13 	Lighting: Any parking area which is intended to be used during non-daylight hours shall be properly illuminated as to avoid accidents. Lights used to illuminate a parking lot shall be arranged and placed at a height and directed and shielded so to direct the light away from adjoining property and roadways so as not be a nuisance or hazard to surrounding uses, occupants, or motorists.

602.14 	Separation from Right-of-Way: Parking facilities located within required front or side yards shall be separated from sidewalks and streets in public right-of-way by a strip of land which shall be at least 5 feet in width, and which shall be reserved and maintained as landscaped open space.

602.15 	Signs: Where necessary due to multiple curb cuts, the entrances, exits, and the intended circulation pattern of the parking area and handicapped parking spaces and fire lanes shall be clearly marked, but only at places and to a height which will not pose a line-of-sight safety hazard. No such signs will require permits, and must be otherwise in accordance with the provisions in Article 7 of this Resolution.

602.16 	Joint Use of Facilities: Two or more non-residential uses may jointly provide and use parking spaces when the operating hours of each use do not normally overlap, provided that the shared use arrangement is provided for within the deeds or other recorded written legal documents approved by the Board of Zoning Appeals.

602.17 	Collective Parking Areas: Two or more non-residential uses may collectively provide the required off-street parking area, provided the required number of parking spaces shall be adequate for the collective uses.

SECTION 603   	PARKING SPACE REQUIREMENTS

For the purposes of this Resolution the following parking space requirements shall apply.

603.1	Residential:

Single Family or two-family = 2 per unit (may include driveway)

Apartment or hotel = 2 per unit

Mobile Home = 2 per unit

Boarding home / Rooming house / Dorm house = 2 per bedroom

603.2	Non-Residential: The number of on-site parking spaces required for all uses not listed in 603.1  shall be determined by the Board of Zoning Appeals. In determining a minimum number of spaces, the BZA shall start with a formula of 1 parking space for every 1.5 persons listed on the facility’s maximum occupancy capacity. The BZA should also factor in the expected hours of operation, availability of local public parking lots, and availability of on-street parking, with the goal of preserving on-street parking for adjacent residential use.

SECTION 604   	LOADING REQUIREMENTS

604.1  	In any district, in connection with every building or part thereof hereafter erected, having a gross floor area of l0,000 square feet or more, which is to be occupied by manufacturing, storage, warehouse, goods display, retail store, wholesale store, market, hotel, hospital, mortuary, laundry, dry cleaning or other uses similarly requiring the receipt 	of distribution by vehicles of material or merchandise there shall be provided and 	maintained on the same lot with such building, at least l off-street loading space plus l 	additional such loading space for each additional 20,000 square feet or major fraction thereof.

604.2  	Each loading space shall be not less than 10 feet in width, 25 feet in length and 14 			feet in height.

604.3  	Such space may occupy all or any part of any required yard space except when abutting any Residential District. When adjacent to any Residential District,  no such space shall be located closer than 50 feet to the shared boundary of any Residential District.

SECTION 605       DRIVEWAY ACCESS REQUIREMENTS

605.1	An approved permit from the township zoning office is required prior to gaining access to the roadway and construction of a driveway.

605.2	Written approval must be obtained by the appropriate road authority:

A.	State Roads – permission comes from Ohio Department of 						Transportation

B.	County Roads – permission comes from Greene County Engineer’s Office

C.	Township Roads – permission comes from Township Road Supervisor

605.3	For driveways connecting to township roads:

A.	The Township Road Supervisor will determine if a culvert is required and what size culvert to be used.

B.	The driveway shall be installed to the satisfaction of the Road Supervisor.

605.4	No driveway shall be located within 10 feet of any property line.  Two or more identical uses may jointly provide and use a shared driveway that abuts or crosses a shared boundary line, provided that the shared use arrangement and access easements are provided for within the deeds or other recorded written legal documents, and approved by the Board of Zoning Appeals.